	UV ink Cartridge (M)

	Safety Data Sheet
	

	according to the REACH Regulation (EC) 1907/2006 amended by Regulation (EU) 2020/878, Regulation (EC) 1272/2008 amended by Regulation (EU) 2024/2865
Reference number: CSSS-TCO-010-173146

Issue date: 14/04/2026   Revision date: 14/04/2026   Version: 1.0

	


	UV ink Cartridge (M)

	Safety Data Sheet
	

	according to the REACH Regulation (EC) 1907/2006 amended by Regulation (EU) 2020/878, Regulation (EC) 1272/2008 amended by Regulation (EU) 2024/2865

	



SECTION 1: Identification of the substance/mixture and of the company/undertaking
1.1. Product identifier
	Product form
	:
	Mixture

	Product name
	:
	UV ink Cartridge (M)

	Additional identification
	:
	Nanoform is NOT covered by this SDS.

	UFI
	:
	N/A

	Product code
	:
	V7260


1.2. Relevant identified uses of the substance or mixture and uses advised against
1.2.1. Relevant identified uses
	Use of the substance/mixture
	:
	Ink for inkjet printing.


1.2.2. Uses advised against
	Restrictions on use
	:
	Not available


1.3. Details of the supplier of the safety data sheet
	Anker Innovations Limited 
Unit 56, 8th Floor, Tower 2, Admiralty Centre, 18 Harcourt Road, Hong Kong
T +86-13631662024, F -
max.yi@anker-in.com


1.4. Emergency telephone number
	Emergency number
	:
	+86-731 8956 6615  24h


SECTION 2: Hazards identification
2.1. Classification of the substance or mixture
Classification according to Regulation (EC) No. 1272/2008 [CLP]
	Acute toxicity (oral), Category 4
	H302 
	

	Skin corrosion/irritation, Category 1, Sub-Category 1C
	H314 
	

	Serious eye damage/eye irritation, Category 1
	H318 
	

	Skin sensitisation, category 1A
	H317 
	

	Reproductive toxicity, Category 1B
	H360Df 
	

	Specific target organ toxicity – Repeated exposure, Category 1
	H372 
	

	Hazardous to the aquatic environment – Acute Hazard, Category 1
	H400 
	

	Hazardous to the aquatic environment – Chronic Hazard, Category 2
	H411 
	


Full text of H- and EUH-statements: see section 16
Adverse physicochemical, human health and environmental effects
May damage fertility or the unborn child. Causes damage to organs through prolonged or repeated exposure. Harmful if swallowed. Causes severe skin burns and eye damage. May cause an allergic skin reaction. Causes serious eye damage. Very toxic to aquatic life. Toxic to aquatic life with long lasting effects.
2.2. Label elements
Labelling according to Regulation (EC) No. 1272/2008 [CLP]
	Hazard pictograms (CLP)
	:
	[image: image1.png]



	[image: image2.png]



	[image: image3.png]



	[image: image4.png]



	
	

	
	
	GHS05
	GHS07
	GHS08
	GHS09
	
	

	Signal word (CLP)
	:
	Danger

	Contains
	:
	Tetrahydrofurfuryl acrylate; Hexamethylene diacrylate; 1-vinylhexahydro-2H-azepin-2-one; Phenylbis(2,4,6-trimethylbenzoyl)phosphine oxide; Exo-1,7,7-trimethylbicyclo[2.2.1]hept-2-yl acrylate / IBOA ; 2-phenoxyethyl acrylate; Glycerol, propoxylated, esters with acrylic acid ; Phenothiazine

	Hazard statements (CLP)
	:
	H302 - Harmful if swallowed.
H314 - Causes severe skin burns and eye damage.
H317 - May cause an allergic skin reaction.
H360Df - May damage the unborn child. Suspected of damaging fertility.
H372 - Causes damage to organs (liver, respiratory tract) through prolonged or repeated exposure.
H410 - Very toxic to aquatic life with long lasting effects.

	Precautionary statements (CLP)
	:
	P201 - Obtain special instructions before use.
P202 - Do not handle until all safety precautions have been read and understood.
P260 - Do not breathe dust/fume/gas/mist/vapours/spray.
P264 - Wash hands, forearms and face thoroughly after handling.
P270 - Do not eat, drink or smoke when using this product.
P272 - Contaminated work clothing should not be allowed out of the workplace.
P273 - Avoid release to the environment.
P280 - Wear protective gloves/protective clothing/eye protection/face protection/hearing protection.
P301+P330+P331 - IF SWALLOWED: Rinse mouth. Do NOT induce vomiting.
P302+P352 - IF ON SKIN: Wash with plenty of water.
P303+P361+P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water .
P304+P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing.
P305+P351+P338+P310 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a POISON CENTER or doctor.
P308+P313 - IF exposed or concerned: Get medical advice/attention.
P314 - Get medical advice/attention if you feel unwell.
P321 - Specific treatment (see supplemental first aid instruction on this label).
P330 - Rinse mouth.
P333+P313 - If skin irritation or rash occurs: Get medical advice/attention.
P362+P364 - Take off contaminated clothing and wash it before reuse.
P363 - Wash contaminated clothing before reuse.
P391 - Collect spillage.
P405 - Store locked up.
P501 - Dispose of contents/container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

	Extra phrases
	:
	Restricted to professional users.


2.3. Other hazards
	This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII

	This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII

	Contains no PBT and/or vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII


	The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or substance(s) are not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %


SECTION 3: Composition/information on ingredients
3.1. Substances
Not applicable
3.2. Mixtures
	Name
	Product identifier
	%
	Classification according to Regulation (EC) No. 1272/2008 [CLP]

	Tetrahydrofurfuryl acrylate
	CAS-No.: 2399-48-6
EC-No.: 219-268-7
REACH-no: 01-2120738396-46-xxxx
	30 – 60
	Acute Tox. 4 (Oral), H302
Skin Corr. 1C, H314
Eye Dam. 1, H318
Skin Sens. 1B, H317
Repr. 1B, H360Df
Aquatic Chronic 2, H411

	1-vinylhexahydro-2H-azepin-2-one
	CAS-No.: 2235-00-9
EC-No.: 218-787-6
REACH-no: 01-2119977109-27-xxxx
	10 – 30
	Acute Tox. 4 (Oral), H302 
Acute Tox. 4 (Dermal), H312 
Eye Irrit. 2, H319
Skin Sens. 1B, H317
STOT RE 1, H372

	Hexamethylene diacrylate
	CAS-No.: 13048-33-4
EC-No.: 235-921-9
EC Index-No.: 607-109-00-8
REACH-no: 01-2119484737-22-xxxx
	10 – 30
	Skin Irrit. 2, H315
Eye Irrit. 2, H319
Skin Sens. 1, H317
Aquatic Acute 1, H400
Aquatic Chronic 2, H411

	Exo-1,7,7-trimethylbicyclo[2.2.1]hept-2-yl acrylate / IBOA 
	CAS-No.: 5888-33-5
EC-No.: 227-561-6
EC Index-No.: 607-756-00-6
REACH-no: 01-2119957862-25-xxxx
	5 – 10
	Skin Sens. 1A, H317
Aquatic Acute 1, H400
Aquatic Chronic 1, H410

	2-phenoxyethyl acrylate
	CAS-No.: 48145-04-6
EC-No.: 256-360-6
REACH-no: 01-2119980532-35-xxxx
	1 – 5
	Skin Sens. 1A, H317
Repr. 2, H361
Aquatic Chronic 2, H411

	Phenylbis(2,4,6-trimethylbenzoyl)phosphine oxide
	CAS-No.: 162881-26-7
EC-No.: 423-340-5
EC Index-No.: 015-189-00-5
REACH-no: 01-2119489401-38-xxxx
	1 – 5
	Skin Sens. 1A, H317
Aquatic Chronic 4, H413

	Oxybis(methyl-2,1-ethanediyl) diacrylate / DPGDA 
	CAS-No.: 57472-68-1
EC-No.: 260-754-3
REACH-no: 01-2119484629-21-xxxx
	1 – 5
	Skin Irrit. 2, H315
Eye Dam. 1, H318
Skin Sens. 1, H317

	Glycerol, propoxylated, esters with acrylic acid 
	CAS-No.: 52408-84-1
EC-No.: 500-114-5

REACH-no: 01-2119487948-12-xxxx
	0,5 –  < 1
	Eye Irrit. 2, H319
Skin Sens. 1, H317

	Poly[oxy(methyl-1,2-ethanediyl)], α,α'-(2,2-dimethyl-1,3-propanediyl)bis[ω-[(1-oxo-2-propenyl)oxy]-
	CAS-No.: 84170-74-1
REACH-no: 01-2119970213-43-xxxx
	0,5 –  < 1
	Skin Sens. 1B, H317
Aquatic Chronic 2, H411

	Phenothiazine
	CAS-No.: 92-84-2
EC-No.: 202-196-5
REACH-no: 01-2119488529-19-xxxx
	0,1 –  < 0,5
	Acute Tox. 4 (Oral), H302 
Skin Sens. 1, H317
STOT RE 2, H373
Aquatic Acute 1, H400
Aquatic Chronic 1, H410

	Tetrahydrofurfuryl alcohol
	CAS-No.: 97-99-4
EC-No.: 202-625-6
EC Index-No.: 603-061-00-7
REACH-no: 01-2119968921-26-xxxx
	< 0,28
	Repr. 1B, H360Df
Eye Irrit. 2, H319


Full text of H- and EUH-statements: see section 16
SECTION 4: First aid measures
4.1. Description of first aid measures
	First-aid measures general
	:
	Call a physician immediately.

	First-aid measures after inhalation
	:
	Remove person to fresh air and keep comfortable for breathing. If breathing stops, give artificial respiration. Get immediate medical advice/attention.

	First-aid measures after skin contact
	:
	Rinse skin with water/shower. Take off immediately all contaminated clothing. Call a physician immediately.

	First-aid measures after eye contact
	:
	Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Call a physician immediately.

	First-aid measures after ingestion
	:
	Rinse mouth. Do not induce vomiting. Call a physician immediately. Never give anything by mouth to an unconscious person.


4.2. Most important symptoms and effects, both acute and delayed
	Symptoms/effects after inhalation
	:
	None under normal conditions.

	Symptoms/effects after skin contact
	:
	Burns. May cause an allergic skin reaction.

	Symptoms/effects after eye contact
	:
	Serious damage to eyes.

	Symptoms/effects after ingestion
	:
	Harmful if swallowed. Burns.

	Chronic symptoms
	:
	May damage fertility or the unborn child.


4.3. Indication of any immediate medical attention and special treatment needed
Treat symptomatically.
SECTION 5: Firefighting measures
5.1. Extinguishing media
	Suitable extinguishing media
	:
	Water spray. Dry powder. Foam. Carbon dioxide.

	Unsuitable extinguishing media
	:
	Not available.


5.2. Special hazards arising from the substance or mixture
	Fire hazard
	:
	No fire hazard.

	Explosion hazard
	:
	No direct explosion hazard.

	Hazardous decomposition products in case of fire
	:
	Toxic fumes may be released.


5.3. Advice for firefighters
	Firefighting instructions
	:
	Fight fire from safe distance and protected location. Do not enter fire area without proper protective equipment, including respiratory protection.

	Protection during firefighting
	:
	Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing.


SECTION 6: Accidental release measures
6.1. Personal precautions, protective equipment and emergency procedures
	General measures
	:
	Stop leak if safe to do so. Notify authorities if product enters sewers or public waters. Absorb spillage to prevent material damage.


6.1.1. For non-emergency personnel
	Protective equipment
	:
	Wear recommended personal protective equipment.

	Emergency procedures
	:
	Only qualified personnel equipped with suitable protective equipment may intervene. Do not breathe dust/fume/gas/mist/vapours/spray. Remove all sources of ignition. Avoid contact with skin, eyes and clothing. Evacuate personnel to a safe area. Keep people away from and upwind of spill/leak.


6.1.2. For emergency responders
	Protective equipment
	:
	Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection".

	Emergency procedures
	:
	Evacuate unnecessary personnel. Stop leak if safe to do so.


6.2. Environmental precautions
Avoid release to the environment. Notify authorities if product enters sewers or public waters. Prevent further leakage or spillage if safe to do so.
6.3. Methods and material for containment and cleaning up
	For containment
	:
	Collect spillage. Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams. Stop leak without risks if possible.

	Methods for cleaning up
	:
	Cover spill with non combustible material, e.g.: sand, earth, vermiculite. Use non-sparking tools.

	Other information
	:
	Dispose of materials or solid residues at an authorized site.


6.4. Reference to other sections
For further information refer to section 13.
SECTION 7: Handling and storage
7.1. Precautions for safe handling
	Additional hazards when processed
	:
	Not expected to present a significant hazard under anticipated conditions of normal use.

	Precautions for safe handling
	:
	Ensure good ventilation of the work station. Obtain special instructions before use. Do not handle until all safety precautions have been read and understood. Wear personal protective equipment. Do not breathe dust/fume/gas/mist/vapours/spray. Avoid contact with skin and eyes.

	Hygiene measures
	:
	Separate working clothes from town clothes. Launder separately. Wash contaminated clothing before reuse. Contaminated work clothing should not be allowed out of the workplace. Do not eat, drink or smoke when using this product. Always wash hands after handling the product.


7.2. Conditions for safe storage, including any incompatibilities
	Technical measures
	:
	Keep in a cool, well-ventilated place away from heat.

	Storage conditions
	:
	Store locked up. Keep container tightly closed and dry. Keep container closed when not in use. Keep out of reach of children. Protect from heat and direct sunlight. Keep away from ignition sources.

	Storage temperature
	:
	5 – 25 °C

	Packaging materials
	:
	Always store product in container of same material as original container.

	Storage class (LGK, TRGS 510)
	:
	LGK 6.1C - Combustible substances of acute toxicity, category 3 / hazardous substances that are toxic or produce chronic effects

	Joint storage table
	:
	[image: image5.png]LGK 4.1B

LGK 6.1D

LGK 4.2 LGK 4.3

LGK 6.1A

LGK 5.1B

LGK 6.1B LGK 6.1C
LGK 8A LGK 8B
LGK 13 LGK 10-13






	Joint storage not permitted for
	:
	LGK 1, LGK 2A, LGK 4.1A, LGK 5.1A, LGK 5.1C, LGK 5.2, LGK 6.2, LGK 7

	Joint storage with restrictions permitted for
	:
	LGK 4.2, LGK 4.3, LGK 5.1B

	Joint storage permitted for
	:
	LGK 2B, LGK 3, LGK 4.1B, LGK 6.1A, LGK 6.1B, LGK 6.1C, LGK 6.1D, LGK 8A, LGK 8B, LGK 10, LGK 11, LGK 12, LGK 13, LGK 10-13


7.3. Specific end use(s)
No additional information available

SECTION 8: Exposure controls/personal protection
8.1. Control parameters
8.1.1 National occupational exposure and biological limit values
	Phenothiazine (92-84-2)

	Belgium - Occupational Exposure Limits

	Local name
	Phénothiazine # Fenothiazine

	OEL TWA
	5 mg/m³

	Remark
	D: la mention “D” signifie que la résorption de l’agent, via la peau, les muqueuses ou les yeux, constitue une partie importante de l’exposition totale. Cette résorption peut se faire tant par contact direct que par présence de l’agent dans l’air. # D: de vermelding “D” betekent dat de opname van het agens via de huid, de slijmvliezen of de ogen een belangrijk deel van de totale blootstelling vormt. Deze opname kan het gevolg zijn van zowel direct contact als zijn aanwezigheid in de lucht.

	Regulatory reference
	Koninklijk besluit/Arrêté royal 16/11/2023

	Denmark - Occupational Exposure Limits

	Local name
	Phenothiazin (Dibenzothiazin)

	8-timers grænseværdi (OEL TWA)
	5 mg/m³

	Korttidsgrænseværdi (OEL STEL)*
	10 mg/m³

	Remark
	H (betyder, at stoffet kan optages gennem huden)

	Regulatory reference
	BEK nr 1356 af 19/11/2025

	Finland - Occupational Exposure Limits

	Local name
	Fenotiatsiini

	HTP (OEL TWA)
	5 mg/m³

	HTP (OEL STEL)
	10 mg/m³

	Remark
	Iho

	Regulatory reference
	HTP-ARVOT 2025 (Sosiaali- ja terveysministeriö)

	France - Occupational Exposure Limits

	Local name
	Phénothiazine

	VLEP 8h (OEL TWA)
	5 mg/m³

	Remark
	Valeurs recommandées/admises. Risque de pénétration percutanée

	Regulatory reference
	Circulaire du Ministère du travail (réf.: INRS ED 6443, 2022; Outil65)

	Greece - Occupational Exposure Limits

	Local name
	Φαινοθειαζίνη

	OEL TWA
	5 mg/m³

	Remark
	Η ένδειξη «δέρμα» στις οριακές τιμές επαγγελματικής έκθεσης επισημαίνει το ενδεχόμενο σημαντικής διείσδυσης μέσω του δέρματος.

	Regulatory reference
	Π.Δ. 90/1999 - Προστασία της υγείας των εργαζομένων που εκτίθενται σε ορισμένους χημικούς παράγοντες κατά τη διάρκεια της εργασίας τους

	Ireland - Occupational Exposure Limits

	Local name
	Phenothiazine

	OEL TWA
	5 mg/m³

	Remark
	Advisory OELV (Advisory Occupational Exposure Limit Values)

	Regulatory reference
	Chemical Agents Code of Practice 2024

	Poland - Occupational Exposure Limits

	Local name
	Dibenzo-1,4-tiazyna

	NDS (OEL TWA)
	4 mg/m³

	Regulatory reference
	Dz. U. 2024 poz. 1017 wraz z późn. zm.

	Portugal - Occupational Exposure Limits

	Local name
	Fenotiazina

	OEL TWA
	5 mg/m³

	Remark
	P (Toxicidade percutânea)

	Regulatory reference
	Norma Portuguesa NP 1796:2014

	Spain - Occupational Exposure Limits

	Local name
	Fenotiazina

	VLA-ED (OEL TWA)
	5 mg/m³

	Remark
	Vía dérmica (Indica que, en las exposiciones a esta sustancia, la aportación por la vía cutánea puede resultar significativa para el contenido corporal total si no se adoptan medidas para prevenir la absorción. En estas situaciones, es aconsejable la utilización del control biológico para poder cuantificar la cantidad global absorbida del contaminante).

	Regulatory reference
	Límites de Exposición Profesional para Agentes Químicos en España 2025. INSHT

	Iceland - Occupational Exposure Limits

	Local name
	Fenóþíasín (díbensóþíasín)

	OEL TWA
	5 mg/m³

	Remark
	H (efnið getur auðveldlega borist inn í líkamann gegnum húð)

	Regulatory reference
	Reglugerð um mengunarmörk og aðgerðir til að draga úr mengun á vinnustöðum (Nr. 390/2009)

	Norway - Occupational Exposure Limits

	Local name
	Fenotiazin

	Grenseverdi (OEL TWA)
	5 mg/m³

	Remark
	H: Kjemikalier som kan tas opp gjennom huden.

	Regulatory reference
	FOR-2025-12-18-2660

	Switzerland - Occupational Exposure Limits

	Local name
	Phénothiazine / Phenothiazin

	MAK (OEL TWA)
	5 mg/m³ (i) / (e)

	Notation
	R / H

	Regulatory reference
	www.suva.ch, 01.01.2026


	Tetrahydrofurfuryl alcohol (97-99-4)

	Austria - Occupational Exposure Limits

	Local name
	Tetrahydrofurfurylalkohol

	Remark
	Fortpflanzungsgefährdend: f, D. Krebserzeugend: 1B

	Regulatory reference
	BGBl. II Nr. 339/2025


8.1.2. Recommended monitoring procedures
No additional information available

8.1.3. Air contaminants formed
No additional information available

8.1.4. DNEL and PNEC
No additional information available

8.1.5. Control banding
No additional information available

8.2. Exposure controls
8.2.1. Appropriate engineering controls
	Appropriate engineering controls:

	Ensure good ventilation of the work station.


8.2.2. Personal protection equipment
	Personal protective equipment:

	Wear recommended personal protective equipment.

	Personal protective equipment symbol(s):

	[image: image6.png]


 [image: image7.png]


 [image: image8.png]





8.2.2.1. Eye and face protection
	Eye protection:

	Safety glasses. Safety glasses with side shields


8.2.2.2. Skin protection
	Skin and body protection:

	Wear suitable protective clothing


	Hand protection:

	Protective gloves. Chemically resistant protective gloves


	Hand protection

	Type
	Material
	Permeation
	Thickness (mm)
	Penetration
	Standard

	
	Nitrile rubber (NBR)
	6 (> 480 minutes)
	0.4
	
	

	
	Chloroprene rubber (CR)
	6 (> 480 minutes)
	0.5
	
	

	
	Polyvinylchloride (PVC)
	6 (> 480 minutes)
	0.7
	
	


8.2.2.3. Respiratory protection
	Respiratory protection:

	[In case of inadequate ventilation] wear respiratory protection.


8.2.2.4. Thermal hazards
No additional information available

8.2.3. Environmental exposure controls
	Environmental exposure controls:

	Avoid release to the environment.


SECTION 9: Physical and chemical properties
9.1. Information on basic physical and chemical properties
	Physical state
	:
	Liquid

	Colour
	:
	Not available

	Odour
	:
	Mild odor. Sweet. acrylic.

	Odour threshold
	:
	Not available

	Melting point
	:
	Not applicable

	Freezing point
	:
	Not available

	Boiling point
	:
	> 149 °C

	Flammability
	:
	Non flammable

	Explosive limits
	:
	Not available

	Lower explosion limit
	:
	Not available

	Upper explosion limit
	:
	Not available

	Flash point
	:
	> 94 °C (PMCC)

	Auto-ignition temperature
	:
	Not available

	Decomposition temperature
	:
	Not available

	pH
	:
	Not available

	Viscosity, kinematic
	:
	Not available

	Solubility
	:
	Not available

	Partition coefficient n-octanol/water (Log Kow)
	:
	Not available

	Vapour pressure
	:
	Not available

	Vapour pressure at 50°C
	:
	Not available

	Density
	:
	Not available

	Relative density
	:
	1,05

	Relative vapour density at 20°C
	:
	Not available

	Particle characteristics
	:
	Not applicable


9.2. Other information
9.2.1. Information with regard to physical hazard classes
No additional information available

9.2.2. Other safety characteristics
No additional information available

SECTION 10: Stability and reactivity
10.1. Reactivity
The product is non-reactive under normal conditions of use, storage and transport.
10.2. Chemical stability
Stable under normal conditions.
10.3. Possibility of hazardous reactions
No dangerous reactions known under normal conditions of use.
10.4. Conditions to avoid
Incompatible materials. Keep out of direct sunlight. Keep away from sources of ignition.
10.5. Incompatible materials
Strong acids. Strong bases. Strong oxidizing agents. Reducing agents.
10.6. Hazardous decomposition products
Thermal decomposition can lead to the release of irritating gases and vapours. Carbon dioxide. Carbon monoxide.
SECTION 11: Toxicological information
11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008
	Acute toxicity (oral)
	:
	Harmful if swallowed.

	Acute toxicity (dermal)
	:
	Not classified

	Acute toxicity (inhalation)
	:
	Not classified


	UV ink Cartridge (M) 

	ATE CLP (oral)
	1091,88 mg/kg

	ATE CLP (dermal)
	5666,67 mg/kg


	Tetrahydrofurfuryl acrylate (2399-48-6)

	LD50 oral rat
	928 mg/kg


	1-vinylhexahydro-2H-azepin-2-one (2235-00-9)

	LD50 oral rat
	1114 mg/kg

	LD50 dermal rabbit
	1700 mg/kg


	Skin corrosion/irritation
	:
	Causes severe skin burns.


	Serious eye damage/irritation
	:
	Causes serious eye damage.


	Respiratory or skin sensitisation
	:
	May cause an allergic skin reaction.


	Germ cell mutagenicity
	:
	Not classified


	Carcinogenicity
	:
	Not classified


	Reproductive toxicity
	:
	May damage the unborn child. Suspected of damaging fertility.


	STOT-single exposure
	:
	Not classified


	STOT-repeated exposure
	:
	Causes damage to organs (liver, respiratory tract) through prolonged or repeated exposure.


	Aspiration hazard
	:
	Not classified


11.2. Information on other hazards
11.2.1. Endocrine disrupting properties
	Adverse health effects caused by endocrine disrupting properties
	:
	The substance/mixture has no endocrine disrupting properties.


11.2.2. Other information
No additional information available

SECTION 12: Ecological information
12.1. Toxicity
	Ecology - general
	:
	Very toxic to aquatic life. Toxic to aquatic life with long lasting effects.

	Hazardous to the aquatic environment, short–term (acute)
	:
	Very toxic to aquatic life.

	Hazardous to the aquatic environment, long–term (chronic)
	:
	Toxic to aquatic life with long lasting effects.


	Tetrahydrofurfuryl acrylate (2399-48-6)

	LC50 96h - Fish [1]
	7,32 mg/l

	EC50 48h - Crustacea [1]
	37,7 mg/l

	EC50 72h - Algae [1]
	3,92 mg/l


	Hexamethylene diacrylate (13048-33-4)

	LC50 96h - Fish [1]
	0,38 mg/l

	EC50 48h - Crustacea [1]
	2,7 mg/l

	EC50 72h - Algae [1]
	1,09 mg/l

	NOEC chronic fish
	0,072 mg/l

	NOEC chronic crustacea
	0,14 mg/l

	NOEC chronic algae
	0,9 mg/l


12.2. Persistence and degradability
No additional information available

12.3. Bioaccumulative potential
No additional information available

12.4. Mobility in soil
No additional information available

12.5. Results of PBT and vPvB assessment
	UV ink Cartridge (M) 

	This substance/mixture does not meet the PBT criteria of REACH regulation, annex XIII

	This substance/mixture does not meet the vPvB criteria of REACH regulation, annex XIII


12.6. Endocrine disrupting properties
	Adverse effects on the environment caused by endocrine disrupting properties
	:
	The substance/mixture has no endocrine disrupting properties.


12.7. Other adverse effects
No additional information available

SECTION 13: Disposal considerations
13.1. Waste treatment methods
	Regional waste regulation
	:
	Disposal must be done according to official regulations.

	Waste treatment methods
	:
	Dispose of contents/container in accordance with licensed collector’s sorting instructions.

	Sewage disposal recommendations
	:
	Disposal must be done according to official regulations.

	Product/Packaging disposal recommendations
	:
	Disposal must be done according to official regulations.

	Additional information
	:
	Do not re-use empty containers.

	Ecological waste information
	:
	The waste of the product should be considered as hazardous as the product itself, with the likelihood of impacting the environment in the same way. Consider the handling and disposal of the waste as defined by the product itself.

	HP Code
	:
	HP5 - “Specific Target Organ Toxicity (STOT)/Aspiration Toxicity:” waste which can cause specific target organ toxicity either from a single or repeated exposure, or which cause acute toxic effects following aspiration.
HP6 - “Acute Toxicity:” waste which can cause acute toxic effects following oral or dermal administration, or inhalation exposure.
HP8 - “Corrosive:” waste which on application can cause skin corrosion.
HP10 - “Toxic for reproduction:” waste which has adverse effects on sexual function and fertility in adult males and females, as well as developmental toxicity in the offspring.
HP13 - “Sensitising:” waste which contains one or more substances known to cause sensitising effects to the skin or the respiratory organs.
HP14 - “Ecotoxic:” waste which presents or may present immediate or delayed risks for one or more sectors of the environment


SECTION 14: Transport information
In accordance with ADR / IMDG / IATA / ADN / RID
	ADR
	IMDG
	IATA
	ADN
	RID

	14.1. UN number or ID number

	UN 1760
	UN 1760
	UN 1760
	UN 1760
	UN 1760

	14.2. UN proper shipping name

	CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate)
	CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate)
	Corrosive liquid, n.o.s. (Tetrahydrofurfuryl acrylate)
	CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate)
	CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate)

	Transport document description

	UN 1760 CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate), 8, III, (E), ENVIRONMENTALLY HAZARDOUS
	UN 1760 CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate), 8, III, MARINE POLLUTANT/ENVIRONMENTALLY HAZARDOUS
	UN 1760 Corrosive liquid, n.o.s. (Tetrahydrofurfuryl acrylate), 8, III, ENVIRONMENTALLY HAZARDOUS
	UN 1760 CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate), 8, III, ENVIRONMENTALLY HAZARDOUS
	UN 1760 CORROSIVE LIQUID, N.O.S. (Tetrahydrofurfuryl acrylate), 8, III, ENVIRONMENTALLY HAZARDOUS

	14.3. Transport hazard class(es)

	8
	8
	8
	8
	8

	[image: image9.png]


[image: image10.png]



	[image: image11.png]


[image: image12.png]



	[image: image13.png]


[image: image14.png]



	[image: image15.png]


[image: image16.png]



	[image: image17.png]


[image: image18.png]




	14.4. Packing group

	III
	III
	III
	III
	III

	14.5. Environmental hazards

	Dangerous for the environment: Yes
	Dangerous for the environment: Yes
Marine pollutant: Yes
	Dangerous for the environment: Yes
	Dangerous for the environment: Yes
	Dangerous for the environment: Yes

	No supplementary information available


14.6. Special precautions for user
	Overland transport

	Classification code (ADR) 
	:
	C9 

	Special provisions (ADR)
	:
	274

	Limited quantities (ADR)
	:
	5l

	Excepted quantities (ADR)
	:
	E1

	Packing instructions (ADR)
	:
	P001, IBC03, LP01, R001

	Mixed packing provisions (ADR)
	:
	MP19 

	Portable tank and bulk container instructions (ADR)
	:
	T7

	Portable tank and bulk container special provisions (ADR)
	:
	TP1, TP28

	Tank code (ADR)
	:
	L4BN

	Vehicle for tank carriage
	:
	AT

	Transport category (ADR)
	:
	3

	Special provisions for carriage - Packages (ADR)
	:
	V12

	Hazard identification number (Kemler No.)
	:
	80 

	Orange plates
	:
	[image: image19.png]




	Tunnel restriction code (ADR)
	:
	E 

	EAC code
	:
	2X 


	Transport by sea

	Special provisions (IMDG)
	:
	223, 274

	Limited quantities (IMDG)
	:
	5 L

	Excepted quantities (IMDG)
	:
	E1

	Packing instructions (IMDG)
	:
	P001, LP01

	IBC packing instructions (IMDG)
	:
	IBC03 

	Tank instructions (IMDG)
	:
	T7 

	Tank special provisions (IMDG)
	:
	TP1, TP28 

	EmS-No. (Fire)
	:
	F-A

	EmS-No. (Spillage)
	:
	S-B

	Stowage category (IMDG)
	:
	A

	Stowage and handling (IMDG)
	:
	SW2

	Properties and observations (IMDG)
	:
	Causes burns to skin, eyes and mucous membranes.


	Air transport

	PCA Excepted quantities (IATA)
	:
	E1

	PCA Limited quantities (IATA)
	:
	Y841

	PCA limited quantity max net quantity (IATA)
	:
	1L 

	PCA packing instructions (IATA)
	:
	852

	PCA max net quantity (IATA)
	:
	5L

	CAO packing instructions (IATA)
	:
	856

	CAO max net quantity (IATA)
	:
	60L

	Special provisions (IATA)
	:
	A3, A803

	ERG code (IATA)
	:
	8L


	Inland waterway transport

	Classification code (ADN)
	:
	C9 

	Special provisions (ADN)
	:
	274

	Limited quantities (ADN)
	:
	5 L

	Excepted quantities (ADN)
	:
	E1

	Carriage permitted (ADN)
	:
	T

	Equipment required (ADN)
	:
	PP, EP

	Number of blue cones/lights (ADN)
	:
	0


	Rail transport

	Classification code (RID)
	:
	C9 

	Special provisions (RID)
	:
	274

	Limited quantities (RID)
	:
	5L

	Excepted quantities (RID)
	:
	E1

	Packing instructions (RID)
	:
	P001, IBC03, LP01, R001

	Mixed packing provisions (RID)
	:
	MP19 

	Portable tank and bulk container instructions (RID)
	:
	T7 

	Portable tank and bulk container special provisions (RID)
	:
	TP1, TP28 

	Tank codes for RID tanks (RID)
	:
	L4BN

	Transport category (RID)
	:
	3

	Special provisions for carriage – Packages (RID)
	:
	W12

	Colis express (express parcels) (RID)
	:
	CE8

	Hazard identification number (RID)
	:
	80


14.7. Maritime transport in bulk according to IMO instruments
Not applicable

SECTION 15: Regulatory information
15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture
15.1.1. EU-Regulations
REACH Annex XVII (Restriction List)
	EU restriction list (REACH Annex XVII)

	Reference code
	Applicable on
	Entry title or description

	3(b)
	UV ink Cartridge (M) ; Tetrahydrofurfuryl acrylate ; Hexamethylene diacrylate ; 1-vinylhexahydro-2H-azepin-2-one ; Exo-1,7,7-trimethylbicyclo[2.2.1]hept-2-yl acrylate / IBOA  ; Glycerol, propoxylated, esters with acrylic acid  ; Tetrahydrofurfuryl alcohol ; Oxybis(methyl-2,1-ethanediyl) diacrylate / DPGDA  ; Poly[oxy(methyl-1,2-ethanediyl)], α,α'-(2,2-dimethyl-1,3-propanediyl)bis[ω-[(1-oxo-2-propenyl)oxy]-
	Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10

	3(c)
	UV ink Cartridge (M) ; Tetrahydrofurfuryl acrylate ; Hexamethylene diacrylate ; Exo-1,7,7-trimethylbicyclo[2.2.1]hept-2-yl acrylate / IBOA  ; Poly[oxy(methyl-1,2-ethanediyl)], α,α'-(2,2-dimethyl-1,3-propanediyl)bis[ω-[(1-oxo-2-propenyl)oxy]-
	Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1

	30.
	Tetrahydrofurfuryl alcohol
	Substances which are classified as reproductive toxicant category 1A or 1B in Part 3 of Annex VI to Regulation (EC) No 1272/2008 and are listed in Appendix 5 or Appendix 6, respectively.


REACH Annex XIV (Authorisation List)
Contains no substance(s) listed on REACH Annex XIV (Authorisation List)
REACH Candidate List (SVHC)
Contains no substance(s) listed on the REACH Candidate List
PIC Regulation (Prior Informed Consent)
Contains no substance(s) listed on the PIC list (Regulation EU 649/2012 concerning the export and import of hazardous chemicals)
POP Regulation (Persistent Organic Pollutants)
Contains no substance(s) listed on the POP list (Regulation EU 2019/1021 on persistent organic pollutants)
Ozone Regulation (1005/2009)
Not listed on the Ozone Depletion list (Regulation EU 2024/590)
Contains no substance(s) listed on the Ozone Depletion list (Regulation EU 2024/590 on substances that deplete the ozone layer)
Explosives Precursors Regulation (EU 2019/1148)
Contains no substance(s) listed on the Explosives Precursors list (Regulation EU 2019/1148 on the marketing and use of explosives precursors)
Drug Precursors Regulation (EC 273/2004)
Contains no substance(s) listed on the Drug Precursors list (Regulation EC 273/2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)
15.1.2. National regulations
	France

	Occupational diseases
Code
Description
RG 65
Eczematiform lesions of allergic mechanism


	Germany

	Employment restrictions
	:
	Observe restrictions according Act on the Protection of Working Mothers (MuSchG).
Observe restrictions according Act on the Protection of Young People in Employment (JArbSchG).

	Water hazard class (WGK)
	:
	WGK 3, Highly hazardous to water (Classification according to AwSV, Annex 1).

	Chemicals Prohibition Ordinance (ChemVerbotsV)
	:
	This product is subject to ChemVerbotsV Annex 2 Entry 1. The following requirements must be observed: authorization requirement (according to § 6 paragraph 1 sentence 1), basic requirements for carrying out the delivery (according to § 8 paragraph 1, 3 and 4), identification and documentation (according to § 9 paragraph 1 to 3) and exclusion of the shipping route (according to § 10).

	Override matching entry (12. BImSchV)
	:
	 Is not subject to the Major Accidents Ordinance (12. BImSchV)

	Netherlands

	ABM category
	:
	Z(1) - non biodegradable substances with hazardous properties for humans and the environment (carcinogenicity/ mutagenicity/ reprotoxicity/bioacumulative potential/ toxicity or persistence)

	SZW-lijst van kankerverwekkende stoffen
	:
	None of the components are listed

	SZW-lijst van mutagene stoffen
	:
	None of the components are listed

	SZW-lijst van reprotoxische stoffen – Borstvoeding
	:
	None of the components are listed

	SZW-lijst van reprotoxische stoffen – Vruchtbaarheid
	:
	Tetrahydrofurfuryl alcohol is listed

	SZW-lijst van reprotoxische stoffen – Ontwikkeling
	:
	Tetrahydrofurfuryl alcohol is listed

	Denmark

	Classification remarks
	:
	Emergency management guidelines for the storage of flammable liquids must be followed

	Danish National Regulations
	:
	Young people below the age of 18 years are not allowed to use the product
Pregnant/breastfeeding women working with the product must not be in direct contact with it. 
If an employee is pregnant or breastfeeding and the person in question uses or is exposed to this product at work, the employer must always carry out a risk assessment of the work. The assessment must both deal with the dangerousness of the impact and its strength and duration. The employer's decision that a pregnant or lactating woman can perform a specific work task must therefore be made in the context of her specific working conditions. See also WEA-Guideline A.1.8-7 on the working environment of pregnant and breastfeeding workers.
The requirements from the Danish Working Environment Authorities regarding work with carcinogens must be followed during use and disposal

	Switzerland

	Storage class (LK)
	:
	LK 6.1 - Toxic materials

	Chemicals Ordinance (ChemO, SR 813.11)
	:
	Group 2


15.2. Chemical safety assessment
No chemical safety assessment has been carried out
SECTION 16: Other information
	Abbreviations and acronyms:

	ACGIH
	American Conference of Governmental Industrial Hygienists

	ADN
	European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways

	ADR
	European Agreement concerning the International Carriage of Dangerous Goods by Road

	ATE
	Acute Toxicity Estimate

	BCF
	Bioconcentration factor

	BLV
	Biological limit value

	BOD
	Biochemical oxygen demand (BOD)

	CAS-No.
	Chemical Abstracts Service number

	CLP
	Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008

	COD
	Chemical oxygen demand (COD)

	CSA
	Chemical safety assessment

	DMEL
	Derived Minimal Effect level

	DNEL
	Derived-No Effect Level

	EC-No.
	European Community number

	EC50
	Median effective concentration

	ED
	Endocrine disruptor

	EN
	European Standard

	EWC
	European waste catalogue

	IARC
	International Agency for Research on Cancer

	IATA
	International Air Transport Association

	IMDG
	International Maritime Dangerous Goods

	LC50
	Median lethal concentration

	LD50
	Median lethal dose

	LOAEL
	Lowest Observed Adverse Effect Level

	Log Kow
	Partition coefficient n-octanol/water (Log Kow)

	Log Pow
	Partition coefficient n-octanol/water (Log Pow)

	MAK
	maximum workplace concentration

	NOAEC
	No-Observed Adverse Effect Concentration

	NOAEL
	No-Observed Adverse Effect Level

	NOEC
	No-Observed Effect Concentration

	N.O.S.
	Not Otherwise Specified

	OECD
	Organisation for Economic Co-operation and Development

	OEL
	Occupational Exposure Limit

	OSHA
	Occupational Safety & Health Administration

	PBT
	Persistent Bioaccumulative Toxic

	PNEC
	Predicted No-Effect Concentration

	PPE
	Personal protection equipment

	RID
	Regulations concerning the International Carriage of Dangerous Goods by Rail

	SDS
	Safety Data Sheet

	STP
	Sewage treatment plant

	TF
	Technical function

	ThOD
	Theoretical oxygen demand (ThOD)

	TLM
	Median Tolerance Limit

	TWA
	Time Weighted Average

	VOC
	Volatile Organic Compounds

	vPvB
	Very Persistent and Very Bioaccumulative

	UFI
	Unique Formula Identifier


	Full text of H- and EUH-statements:

	Acute Tox. 4 (Dermal)
	Acute toxicity (dermal), Category 4

	Acute Tox. 4 (Oral)
	Acute toxicity (oral), Category 4

	Aquatic Acute 1
	Hazardous to the aquatic environment – Acute Hazard, Category 1

	Aquatic Chronic 1
	Hazardous to the aquatic environment – Chronic Hazard, Category 1

	Aquatic Chronic 2
	Hazardous to the aquatic environment – Chronic Hazard, Category 2

	Aquatic Chronic 4
	Hazardous to the aquatic environment – Chronic Hazard, Category 4

	Eye Dam. 1
	Serious eye damage/eye irritation, Category 1

	Eye Irrit. 2
	Serious eye damage/eye irritation, Category 2

	Repr. 1B
	Reproductive toxicity, Category 1B

	Repr. 2
	Reproductive toxicity, Category 2

	Skin Corr. 1C
	Skin corrosion/irritation, Category 1, Sub-Category 1C

	Skin Irrit. 2
	Skin corrosion/irritation, Category 2

	Skin Sens. 1
	Skin sensitisation, Category 1

	Skin Sens. 1A
	Skin sensitisation, category 1A

	Skin Sens. 1B
	Skin sensitisation, category 1B

	STOT RE 1
	Specific target organ toxicity – Repeated exposure, Category 1

	STOT RE 2
	Specific target organ toxicity – Repeated exposure, Category 2

	H302
	Harmful if swallowed.

	H312
	Harmful in contact with skin.

	H314
	Causes severe skin burns and eye damage.

	H315
	Causes skin irritation.

	H317
	May cause an allergic skin reaction.

	H318
	Causes serious eye damage.

	H319
	Causes serious eye irritation.

	H360Df
	May damage the unborn child. Suspected of damaging fertility.

	H361
	Suspected of damaging fertility or the unborn child.

	H372
	Causes damage to organs (liver, respiratory tract) through prolonged or repeated exposure.

	H373
	May cause damage to organs through prolonged or repeated exposure.

	H400
	Very toxic to aquatic life.

	H410
	Very toxic to aquatic life with long lasting effects.

	H411
	Toxic to aquatic life with long lasting effects.

	H413
	May cause long lasting harmful effects to aquatic life.


	The classification complies with
	:
	ATP 12


SDS-EU
This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.
	
	
	

	14/04/2026 (Revision date)
	EN (English)
	1/17


	
	
	

	14/04/2026 (Revision date)
	EN (English)
	17/17



